Home automation using an Arduino Uno with a Bluetooth module HC-05 is a popular and effective way to control various devices remotely. Here's a brief overview of how to set up such a system, including controlling devices like an RC fan, buzzer, water pump, and LED using relays.

### Components Required:

- Arduino Uno: The microcontroller board that controls the automation system.

- HC-05 Bluetooth Module: Provides Bluetooth communication between your Arduino and a smartphone or computer.

- Relay Module: Used to control high-voltage devices like fans, pumps, and other appliances. Each relay can switch on or off an external device.

- RC Fan: A fan typically used in radio-controlled vehicles; it can be controlled through relays.

- Buzzer: An audio signaling device that can be turned on or off.

- Water Pump: A device for pumping water, controlled by a relay.

- LED: A simple light-emitting diode that can be turned on or off.

- Jumper Wires: To connect the components.

### Wiring:

- HC-05 Bluetooth Module:

  - TXD (Transmit): Connect to Arduino's RX (Pin 0).

  - RXD (Receive): Connect to Arduino's TX (Pin 1).

  - VCC: Connect to Arduino's 5V.

  - GND: Connect to Arduino's GND.

- Relay Module:

  - IN1, IN2, IN3, IN4: Connect to Arduino's digital pins (e.g., Pins 2, 3, 4, and 5).

  - VCC: Connect to Arduino's 5V.

  - GND: Connect to Arduino's GND.

  - COM (Common): Connect to the power source of the device.

  - NO (Normally Open): Connect to the device you want to control.

- Devices:

  - RC Fan: Connect to one of the relay's NO and COM terminals.

  - Buzzer: Connect to another relay's NO and COM terminals.

  - Water Pump: Connect to a third relay's NO and COM terminals.

  - LED: Connect to the remaining relay's NO and COM terminals.

### Controlling Devices with a Bluetooth App:

- Arduino Bluetooth Apps:

  - Bluetooth Terminal Apps: Apps like "Bluetooth Terminal" or "Serial Bluetooth Terminal" can be used to send commands from a smartphone to the HC-05 module. Commands like 1, 2, 3, etc., are sent to control different devices.

  - Custom Bluetooth Apps: You can use apps like "Bluetooth Electronics" or "Arduino Bluetooth Controller" to create a custom interface with buttons to control each device.

- Control Methods:

  - Keyboard Input: Enter commands directly into the terminal app or a custom app to control devices.

  - Buttons: Use a custom app with buttons mapped to specific commands (1 for RC Fan ON, 2 for RC Fan OFF, etc.).

  - Voice Commands: If using an advanced app or integration with voice assistants, configure voice commands to send appropriate Bluetooth commands to the HC-05 module.

Component:-

| Component | Specification | Quantity |
| --- | --- | --- |
| Arduino UNO | Microcontroller Board | 1 |
| Relay Module | 4 relays, 5V control | 1 |
| HC-05 Bluetooth Module | Bluetooth Serial Communication Module | 1 |
| DC Power Jack Adapter Connector | For connecting to 9V battery | 1 |
| Battery | Standard 9V battery | 1 |
| Mini Breadboard | Small prototype board | 1 |
| DC 3-6V Mini Micro Submersible Water Pump | 3-6V Submersible Pump | 1 |
| Cooling Fan | 12V Cooling Fan, 30mm x 10mm | 2 |
| LED | 5mm Blue LED | 1 |
| Male to Female Jumper Wires | 20cm long, male to female | 20(PCS) 1 PACK |
| Male to Male Jumper Wires | 20cm long, male to male | 9(PCS) 1PACK |
| DC Power Jack Adapter | For connecting to external power supply | 1 |

Circuit Diagram :-

HC 05 Bluetooth Module to Arduino UNO

| HC 05 Bluetooth Module | Arduino UNO |
| --- | --- |
| VCC | 5v |
| GND | GND |
| TX | RX(0) |
| RX | TX(1) |

5v 4 Chanel Relay Module to Arduino UNO

| Relay Module | Arduino UNO |
| --- | --- |
| VCC | 5V |
| GND | GND |
| IN1 | D4 |
| IN2 | D5 |
| IN3 | D6 |
| IN4 | D7 |

Code:-

#include <Arduino.h>

#include <Wire.h>

#include <SoftwareSerial.h>

int Livingroom = 4;

int kitchen = 7;

int Diningroom = 6;

int Garden = 5;

SoftwareSerial Bluetooth(0, 1);

char Data;

void sendData(String transmitData){

Bluetooth.println(transmitData);}

void setup(){

Bluetooth.begin(9600);

pinMode(Livingroom,OUTPUT);

pinMode(kitchen,OUTPUT);

pinMode(Diningroom,OUTPUT);

pinMode(Garden,OUTPUT);

}

void loop(){

if(Bluetooth.available()){

Data=Bluetooth.read();

if(Data==('5')){

digitalWrite(Livingroom,1);

sendData(" Living Room Buzzer OFF");

}

if(Data==('1')){

digitalWrite(Livingroom,0);

sendData("Living Room Buzzer ON");

}

if(Data==('6')){

digitalWrite(kitchen,1);

sendData("kitchen Fan OFF");

}

if(Data==('2')){

digitalWrite(kitchen,0);

sendData("Kitchen Fan ON");

}

if(Data==('7')){

digitalWrite(Diningroom,1);

sendData("Dining Room Light OFF");

}

if(Data==('3')){

digitalWrite(Diningroom,0);

sendData("Dining Room Light ON");

}

if(Data==('8')){

digitalWrite(Garden,1);

sendData("Water Pump OFF");

}

if(Data==('4')){

digitalWrite(Garden,0);

sendData("Water Pump  ON");

}

if(Data==('9')){

digitalWrite(Livingroom,1);

digitalWrite(kitchen,1);

digitalWrite(Diningroom,1);

digitalWrite(Garden,1);

sendData("ALL DEVICE'S OFF");

}

if(Data==('0')){

digitalWrite(Livingroom,0);

digitalWrite(kitchen,0);

digitalWrite(Diningroom,0);

digitalWrite(Garden,0);

sendData("ALL DEVICE'S ON");

}

}

}

### Key Points:

- Safety: Ensure that the relays are rated for the voltage and current of the devices you're controlling to avoid any electrical hazards.

- Bluetooth Communication: The HC-05 module allows easy wireless control via a smartphone or computer, making the automation system user-friendly.

- Expandability: This basic setup can be expanded with additional devices and more complex control schemes as needed.

This setup provides a robust and flexible home automation solution using Arduino and Bluetooth.